PROJECT REPORT

ON

Traffic Management System

REPORT SUBMITTED

TO

VISHWAKARMA INSTITUTE OF INFORMATION TECHNOLOGY, PUNE

FOR THE PBL OF PYTHON FOR ENGINEERS

IN

ENGINEERING AND APPLIED SCIENCE DEPARTMENT

By

Mihir Patwardhan- 268/22111346

Aditya Adinath Patil - 269/ 22110016

Krishna Gupta- 270/22110152

Triambak Kapoor – 271/22110153

Class: F.Y BTECH 		 	Division: b			Batch: b3

Batch teacher-: Aparna Barbadekar

INDEX

| Sr. No. | Contents | Page no. |
| --- | --- | --- |
| 1 | Source Code | 03 – 16 |
| 2 | Output | 17 |

Source Code

import random
import time
import threading
import pygame
import sys


# Default values of signal timers
defaultGreen = {0: 10, 1: 10, 2: 10, 3: 10}
defaultRed = 50
defaultYellow = 5

signals = []
noOfSignals = 4
currentGreen = 0  # Indicates which signal is green currently
nextGreen = (currentGreen + 1) % noOfSignals  # Indicates which signal will turn green next
currentYellow = 0  # Indicates whether yellow signal is on or off

speeds = {'car': 1.0, 'bus': 0.6, 'truck': 0.3, 'bike': 1.5}  # average speeds of vehicles

# Coordinates of vehicles' start
x = {'right': [0, 0, 0], 'down': [755, 727, 697], 'left': [1400, 1400, 1400], 'up': [602, 627, 657]}
y = {'right': [348, 370, 398], 'down': [0, 0, 0], 'left': [498, 466, 436], 'up': [800, 800, 800]}

vehicles = {'right': {0: [], 1: [], 2: [], 'crossed': 0}, 'down': {0: [], 1: [], 2: [], 'crossed': 0},
            'left': {0: [], 1: [], 2: [], 'crossed': 0}, 'up': {0: [], 1: [], 2: [], 'crossed': 0}}
vehicleTypes = {0: 'car', 1: 'bus', 2: 'truck', 3: 'bike'}
directionNumbers = {0: 'right', 1: 'down', 2: 'left', 3: 'up'}

# Coordinates of signal image, timer, and vehicle count
signalCoods = [(530, 230), (810, 230), (810, 570), (530, 570)]
signalTimerCoods = [(530, 210), (810, 210), (810, 550), (530, 550)]

# Coordinates of stop lines
stopLines = {'right': 590, 'down': 330, 'left': 800, 'up': 535}
defaultStop = {'right': 580, 'down': 320, 'left': 810, 'up': 545}

# Gap between vehicles
stoppingGap = 25  # stopping gap
movingGap = 25  # moving gap

# set allowed vehicle types here
allowedVehicleTypes = {'car': True, 'bus': True, 'truck': True, 'bike': True}
allowedVehicleTypesList = []
vehiclesTurned = {'right': {1: [], 2: []}, 'down': {1: [], 2: []}, 'left': {1: [], 2: []}, 'up': {1: [], 2: []}}
vehiclesNotTurned = {'right': {1: [], 2: []}, 'down': {1: [], 2: []}, 'left': {1: [], 2: []}, 'up': {1: [], 2: []}}
rotationAngle = 3
mid = {'right': {'x': 705, 'y': 445}, 'down': {'x': 695, 'y': 450}, 'left': {'x': 695, 'y': 425},
       'up': {'x': 695, 'y': 400}}


randomGreenSignalTimer = True
randomGreenSignalTimerRange = [10, 15]

timeElapsed = 0
simulationTime = 300
timeElapsedCoods = (1100, 50)
vehicleCountTexts = ["0", "0", "0", "0"]
vehicleCountCoods = [(480, 210), (880, 210), (880, 550), (480, 550)]

pygame.init()
simulation = pygame.sprite.Group()


class TrafficSignal:
    def __init__(self, red, yellow, green):
        self.red = red
        self.yellow = yellow
        self.green = green
        self.signalText = ""


class Vehicle(pygame.sprite.Sprite):
    def __init__(self, lane, vehicleClass, direction_number, direction, will_turn):
        pygame.sprite.Sprite.__init__(self)
        self.lane = lane
        self.vehicleClass = vehicleClass
        self.speed = speeds[vehicleClass]
        self.direction_number = direction_number
        self.direction = direction
        self.x = x[direction][lane]
        self.y = y[direction][lane]
        self.crossed = 0
        self.willTurn = will_turn
        self.turned = 0
        self.rotateAngle = 0
        vehicles[direction][lane].append(self)
        self.index = len(vehicles[direction][lane]) - 1
        self.crossedIndex = 0
        path = "images/" + direction + "/" + vehicleClass + ".png"
        self.originalImage = pygame.image.load(path)
        self.image = pygame.image.load(path)

        if (len(vehicles[direction][lane]) > 1 and vehicles[direction][lane][self.index - 1].crossed == 0):
            if (direction == 'right'):
                self.stop = vehicles[direction][lane][self.index - 1].stop

            elif (direction == 'left'):
                self.stop = vehicles[direction][lane][self.index - 1].stop

            elif (direction == 'down'):
                self.stop = vehicles[direction][lane][self.index - 1].stop

            elif (direction == 'up'):
                self.stop = vehicles[direction][lane][self.index - 1].stop

        else:
            self.stop = defaultStop[direction]

        # Set new starting and stopping coordinate
        if (direction == 'right'):
            temp = self.image.get_rect().width + stoppingGap
            x[direction][lane] -= temp
        elif (direction == 'left'):
            temp = self.image.get_rect().width + stoppingGap
            x[direction][lane] += temp
        elif (direction == 'down'):
            temp = self.image.get_rect().height + stoppingGap
            y[direction][lane] -= temp
        elif (direction == 'up'):
            temp = self.image.get_rect().height + stoppingGap
            y[direction][lane] += temp
        simulation.add(self)

    def render(self, screen):
        screen.blit(self.image, (self.x, self.y))

    def move(self):
        if (self.direction == 'right'):
            if (self.crossed == 0 and self.x + self.image.get_rect().width > stopLines[self.direction]):
                self.crossed = 1
                vehicles[self.direction]['crossed'] += 1
                if (self.willTurn == 0):
                    vehiclesNotTurned[self.direction][self.lane].append(self)
                    self.crossedIndex = len(vehiclesNotTurned[self.direction][self.lane]) - 1
            if (self.willTurn == 1):
                if (self.lane == 1):
                    if (self.crossed == 0 or self.x + self.image.get_rect().width < stopLines[self.direction] + 40):
                        if ((self.x + self.image.get_rect().width <= self.stop or (
                                currentGreen == 0 and currentYellow == 0) or self.crossed == 1) and (
                                self.index == 0 or self.x + self.image.get_rect().width < (
                                vehicles[self.direction][self.lane][self.index - 1].x - movingGap) or
                                vehicles[self.direction][self.lane][self.index - 1].turned == 1)):
                            self.x += self.speed
                    else:
                        if (self.turned == 0):
                            self.rotateAngle += rotationAngle
                            self.image = pygame.transform.rotate(self.originalImage, self.rotateAngle)
                            self.x += 2.4
                            self.y -= 2.8
                            if (self.rotateAngle == 90):
                                self.turned = 1
                                vehiclesTurned[self.direction][self.lane].append(self)
                                self.crossedIndex = len(vehiclesTurned[self.direction][self.lane]) - 1
                        else:
                            if (self.crossedIndex == 0 or (self.y > (
                                    vehiclesTurned[self.direction][self.lane][self.crossedIndex - 1].y +
                                    vehiclesTurned[self.direction][self.lane][
                                        self.crossedIndex - 1].image.get_rect().height + movingGap))):
                                self.y -= self.speed
                elif (self.lane == 2):
                    if (self.crossed == 0 or self.x + self.image.get_rect().width < mid[self.direction]['x']):
                        if ((self.x + self.image.get_rect().width <= self.stop or (
                                currentGreen == 0 and currentYellow == 0) or self.crossed == 1) and (
                                self.index == 0 or self.x + self.image.get_rect().width < (
                                vehicles[self.direction][self.lane][self.index - 1].x - movingGap) or
                                vehicles[self.direction][self.lane][self.index - 1].turned == 1)):
                            self.x += self.speed
                    else:
                        if (self.turned == 0):
                            self.rotateAngle += rotationAngle
                            self.image = pygame.transform.rotate(self.originalImage, -self.rotateAngle)
                            self.x += 2
                            self.y += 1.8
                            if (self.rotateAngle == 90):
                                self.turned = 1
                                vehiclesTurned[self.direction][self.lane].append(self)
                                self.crossedIndex = len(vehiclesTurned[self.direction][self.lane]) - 1
                        else:
                            if (self.crossedIndex == 0 or ((self.y + self.image.get_rect().height) < (
                                    vehiclesTurned[self.direction][self.lane][self.crossedIndex - 1].y - movingGap))):
                                self.y += self.speed
            else:
                if (self.crossed == 0):
                    if ((self.x + self.image.get_rect().width <= self.stop or (
                            currentGreen == 0 and currentYellow == 0)) and (
                            self.index == 0 or self.x + self.image.get_rect().width < (
                            vehicles[self.direction][self.lane][self.index - 1].x - movingGap))):
                        self.x += self.speed
                else:
                    if ((self.crossedIndex == 0) or (self.x + self.image.get_rect().width < (
                            vehiclesNotTurned[self.direction][self.lane][self.crossedIndex - 1].x - movingGap))):
                        self.x += self.speed
        elif (self.direction == 'down'):
            if (self.crossed == 0 and self.y + self.image.get_rect().height > stopLines[self.direction]):
                self.crossed = 1
                vehicles[self.direction]['crossed'] += 1
                if (self.willTurn == 0):
                    vehiclesNotTurned[self.direction][self.lane].append(self)
                    self.crossedIndex = len(vehiclesNotTurned[self.direction][self.lane]) - 1
            if (self.willTurn == 1):
                if (self.lane == 1):
                    if (self.crossed == 0 or self.y + self.image.get_rect().height < stopLines[self.direction] + 50):
                        if ((self.y + self.image.get_rect().height <= self.stop or (
                                currentGreen == 1 and currentYellow == 0) or self.crossed == 1) and (
                                self.index == 0 or self.y + self.image.get_rect().height < (
                                vehicles[self.direction][self.lane][self.index - 1].y - movingGap) or
                                vehicles[self.direction][self.lane][self.index - 1].turned == 1)):
                            self.y += self.speed
                    else:
                        if (self.turned == 0):
                            self.rotateAngle += rotationAngle
                            self.image = pygame.transform.rotate(self.originalImage, self.rotateAngle)
                            self.x += 1.2
                            self.y += 1.8
                            if (self.rotateAngle == 90):
                                self.turned = 1
                                vehiclesTurned[self.direction][self.lane].append(self)
                                self.crossedIndex = len(vehiclesTurned[self.direction][self.lane]) - 1
                        else:
                            if (self.crossedIndex == 0 or ((self.x + self.image.get_rect().width) < (
                                    vehiclesTurned[self.direction][self.lane][self.crossedIndex - 1].x - movingGap))):
                                self.x += self.speed
                elif (self.lane == 2):
                    if (self.crossed == 0 or self.y + self.image.get_rect().height < mid[self.direction]['y']):
                        if ((self.y + self.image.get_rect().height <= self.stop or (
                                currentGreen == 1 and currentYellow == 0) or self.crossed == 1) and (
                                self.index == 0 or self.y + self.image.get_rect().height < (
                                vehicles[self.direction][self.lane][self.index - 1].y - movingGap) or
                                vehicles[self.direction][self.lane][self.index - 1].turned == 1)):
                            self.y += self.speed
                    else:
                        if (self.turned == 0):
                            self.rotateAngle += rotationAngle
                            self.image = pygame.transform.rotate(self.originalImage, -self.rotateAngle)
                            self.x -= 2.5
                            self.y += 2
                            if (self.rotateAngle == 90):
                                self.turned = 1
                                vehiclesTurned[self.direction][self.lane].append(self)
                                self.crossedIndex = len(vehiclesTurned[self.direction][self.lane]) - 1
                        else:
                            if (self.crossedIndex == 0 or (self.x > (
                                    vehiclesTurned[self.direction][self.lane][self.crossedIndex - 1].x +
                                    vehiclesTurned[self.direction][self.lane][
                                        self.crossedIndex - 1].image.get_rect().width + movingGap))):
                                self.x -= self.speed
            else:
                if (self.crossed == 0):
                    if ((self.y + self.image.get_rect().height <= self.stop or (
                            currentGreen == 1 and currentYellow == 0)) and (
                            self.index == 0 or self.y + self.image.get_rect().height < (
                            vehicles[self.direction][self.lane][self.index - 1].y - movingGap))):
                        self.y += self.speed
                else:
                    if ((self.crossedIndex == 0) or (self.y + self.image.get_rect().height < (
                            vehiclesNotTurned[self.direction][self.lane][self.crossedIndex - 1].y - movingGap))):
                        self.y += self.speed
        elif (self.direction == 'left'):
            if (self.crossed == 0 and self.x < stopLines[self.direction]):
                self.crossed = 1
                vehicles[self.direction]['crossed'] += 1
                if (self.willTurn == 0):
                    vehiclesNotTurned[self.direction][self.lane].append(self)
                    self.crossedIndex = len(vehiclesNotTurned[self.direction][self.lane]) - 1
            if (self.willTurn == 1):
                if (self.lane == 1):
                    if (self.crossed == 0 or self.x > stopLines[self.direction] - 70):
                        if ((self.x >= self.stop or (
                                currentGreen == 2 and currentYellow == 0) or self.crossed == 1) and (
                                self.index == 0 or self.x > (vehicles[self.direction][self.lane][self.index - 1].x +
                                                             vehicles[self.direction][self.lane][
                                                                 self.index - 1].image.get_rect().width + movingGap) or
                                vehicles[self.direction][self.lane][self.index - 1].turned == 1)):
                            self.x -= self.speed
                    else:
                        if (self.turned == 0):
                            self.rotateAngle += rotationAngle
                            self.image = pygame.transform.rotate(self.originalImage, self.rotateAngle)
                            self.x -= 1
                            self.y += 1.2
                            if (self.rotateAngle == 90):
                                self.turned = 1
                                vehiclesTurned[self.direction][self.lane].append(self)
                                self.crossedIndex = len(vehiclesTurned[self.direction][self.lane]) - 1
                        else:
                            if (self.crossedIndex == 0 or ((self.y + self.image.get_rect().height) < (
                                    vehiclesTurned[self.direction][self.lane][self.crossedIndex - 1].y - movingGap))):
                                self.y += self.speed
                elif (self.lane == 2):
                    if (self.crossed == 0 or self.x > mid[self.direction]['x']):
                        if ((self.x >= self.stop or (
                                currentGreen == 2 and currentYellow == 0) or self.crossed == 1) and (
                                self.index == 0 or self.x > (vehicles[self.direction][self.lane][self.index - 1].x +
                                                             vehicles[self.direction][self.lane][
                                                                 self.index - 1].image.get_rect().width + movingGap) or
                                vehicles[self.direction][self.lane][self.index - 1].turned == 1)):
                            self.x -= self.speed
                    else:
                        if (self.turned == 0):
                            self.rotateAngle += rotationAngle
                            self.image = pygame.transform.rotate(self.originalImage, -self.rotateAngle)
                            self.x -= 1.8
                            self.y -= 2.5
                            if (self.rotateAngle == 90):
                                self.turned = 1
                                vehiclesTurned[self.direction][self.lane].append(self)
                                self.crossedIndex = len(vehiclesTurned[self.direction][self.lane]) - 1
                        else:
                            if (self.crossedIndex == 0 or (self.y > (
                                    vehiclesTurned[self.direction][self.lane][self.crossedIndex - 1].y +
                                    vehiclesTurned[self.direction][self.lane][
                                        self.crossedIndex - 1].image.get_rect().height + movingGap))):
                                self.y -= self.speed
            else:
                if (self.crossed == 0):
                    if ((self.x >= self.stop or (currentGreen == 2 and currentYellow == 0)) and (
                            self.index == 0 or self.x > (
                            vehicles[self.direction][self.lane][self.index - 1].x + vehicles[self.direction][self.lane][
                        self.index - 1].image.get_rect().width + movingGap))):
                        self.x -= self.speed
                else:
                    if ((self.crossedIndex == 0) or (self.x > (
                            vehiclesNotTurned[self.direction][self.lane][self.crossedIndex - 1].x +
                            vehiclesNotTurned[self.direction][self.lane][
                                self.crossedIndex - 1].image.get_rect().width + movingGap))):
                        self.x -= self.speed
        elif (self.direction == 'up'):
            if (self.crossed == 0 and self.y < stopLines[self.direction]):
                self.crossed = 1
                vehicles[self.direction]['crossed'] += 1
                if (self.willTurn == 0):
                    vehiclesNotTurned[self.direction][self.lane].append(self)
                    self.crossedIndex = len(vehiclesNotTurned[self.direction][self.lane]) - 1
            if (self.willTurn == 1):
                if (self.lane == 1):
                    if (self.crossed == 0 or self.y > stopLines[self.direction] - 60):
                        if ((self.y >= self.stop or (
                                currentGreen == 3 and currentYellow == 0) or self.crossed == 1) and (
                                self.index == 0 or self.y > (vehicles[self.direction][self.lane][self.index - 1].y +
                                                             vehicles[self.direction][self.lane][
                                                                 self.index - 1].image.get_rect().height + movingGap) or
                                vehicles[self.direction][self.lane][self.index - 1].turned == 1)):
                            self.y -= self.speed
                    else:
                        if (self.turned == 0):
                            self.rotateAngle += rotationAngle
                            self.image = pygame.transform.rotate(self.originalImage, self.rotateAngle)
                            self.x -= 2
                            self.y -= 1.2
                            if (self.rotateAngle == 90):
                                self.turned = 1
                                vehiclesTurned[self.direction][self.lane].append(self)
                                self.crossedIndex = len(vehiclesTurned[self.direction][self.lane]) - 1
                        else:
                            if (self.crossedIndex == 0 or (self.x > (
                                    vehiclesTurned[self.direction][self.lane][self.crossedIndex - 1].x +
                                    vehiclesTurned[self.direction][self.lane][
                                        self.crossedIndex - 1].image.get_rect().width + movingGap))):
                                self.x -= self.speed
                elif (self.lane == 2):
                    if (self.crossed == 0 or self.y > mid[self.direction]['y']):
                        if ((self.y >= self.stop or (
                                currentGreen == 3 and currentYellow == 0) or self.crossed == 1) and (
                                self.index == 0 or self.y > (vehicles[self.direction][self.lane][self.index - 1].y +
                                                             vehicles[self.direction][self.lane][
                                                                 self.index - 1].image.get_rect().height + movingGap) or
                                vehicles[self.direction][self.lane][self.index - 1].turned == 1)):
                            self.y -= self.speed
                    else:
                        if (self.turned == 0):
                            self.rotateAngle += rotationAngle
                            self.image = pygame.transform.rotate(self.originalImage, -self.rotateAngle)
                            self.x += 1
                            self.y -= 1
                            if (self.rotateAngle == 90):
                                self.turned = 1
                                vehiclesTurned[self.direction][self.lane].append(self)
                                self.crossedIndex = len(vehiclesTurned[self.direction][self.lane]) - 1
                        else:
                            if (self.crossedIndex == 0 or (self.x < (
                                    vehiclesTurned[self.direction][self.lane][self.crossedIndex - 1].x -
                                    vehiclesTurned[self.direction][self.lane][
                                        self.crossedIndex - 1].image.get_rect().width - movingGap))):
                                self.x += self.speed
            else:
                if (self.crossed == 0):
                    if ((self.y >= self.stop or (currentGreen == 3 and currentYellow == 0)) and (
                            self.index == 0 or self.y > (
                            vehicles[self.direction][self.lane][self.index - 1].y + vehicles[self.direction][self.lane][
                        self.index - 1].image.get_rect().height + movingGap))):
                        self.y -= self.speed
                else:
                    if ((self.crossedIndex == 0) or (self.y > (
                            vehiclesNotTurned[self.direction][self.lane][self.crossedIndex - 1].y +
                            vehiclesNotTurned[self.direction][self.lane][
                                self.crossedIndex - 1].image.get_rect().height + movingGap))):
                        self.y -= self.speed


# Initialization of signals with default values
def initialize():
    minTime = randomGreenSignalTimerRange[0]
    maxTime = randomGreenSignalTimerRange[1]
    if (randomGreenSignalTimer):
        ts1 = TrafficSignal(0, defaultYellow, random.randint(minTime, maxTime))
        signals.append(ts1)
        ts2 = TrafficSignal(ts1.yellow + ts1.green, defaultYellow, random.randint(minTime, maxTime))
        signals.append(ts2)
        ts3 = TrafficSignal(defaultRed, defaultYellow, random.randint(minTime, maxTime))
        signals.append(ts3)
        ts4 = TrafficSignal(defaultRed, defaultYellow, random.randint(minTime, maxTime))
        signals.append(ts4)
    repeat()

def repeat():
    global currentGreen, currentYellow, nextGreen
    while (signals[currentGreen].green > 0):  # while the timer of current green signal is not zero
        updateValues()
        time.sleep(1)
    currentYellow = 1  # set yellow signal on
    # reset stop coordinates of lanes and vehicles
    for i in range(0, 3):
        for vehicle in vehicles[directionNumbers[currentGreen]][i]:
            vehicle.stop = defaultStop[directionNumbers[currentGreen]]
    while (signals[currentGreen].yellow > 0):  # while the timer of current yellow signal is not zero
        updateValues()
        time.sleep(1)
    currentYellow = 0  # set yellow signal off

    # reset all signal times of current signal to default/random times
    if (randomGreenSignalTimer):
        signals[currentGreen].green = random.randint(randomGreenSignalTimerRange[0], randomGreenSignalTimerRange[1])
    else:
        signals[currentGreen].green = defaultGreen[currentGreen]
    signals[currentGreen].yellow = defaultYellow
    signals[currentGreen].red = defaultRed

    currentGreen = nextGreen  # set next signal as green signal
    nextGreen = (currentGreen + 1) % noOfSignals  # set next green signal
    signals[nextGreen].red = signals[currentGreen].yellow + signals[
        currentGreen].green  # set the red time of next to next signal as (yellow time + green time) of next signal
    repeat()


# Update values of the signal timers after every second
def updateValues():
    for i in range(0, noOfSignals):
        if (i == currentGreen):
            if (currentYellow == 0):
                signals[i].green -= 1
            else:
                signals[i].yellow -= 1
        else:
            signals[i].red -= 1


# Generating vehicles in the simulation
def generateVehicles():
    while (True):
        vehicle_type = random.choice(allowedVehicleTypesList)
        lane_number = random.randint(1, 2)
        will_turn = 0
        if (lane_number == 1):
            temp = random.randint(0, 99)
            if (temp < 40):
                will_turn = 1
        elif (lane_number == 2):
            temp = random.randint(0, 99)
            if (temp < 40):
                will_turn = 1
        temp = random.randint(0, 99)

        dist = [25, 50, 75, 100]
        if (temp < dist[0]):
            direction_number = 0
        elif (temp < dist[1]):
            direction_number = 1
        elif (temp < dist[2]):
            direction_number = 2
        elif (temp < dist[3]):
            direction_number = 3
        Vehicle(lane_number, vehicleTypes[vehicle_type], direction_number, directionNumbers[direction_number],
                will_turn)
        time.sleep(1)


def showStats():
    totalVehicles = 0
    for i in range(0, 4):
        if (signals[i] != None):
            totalVehicles += vehicles[directionNumbers[i]]['crossed']
    print('Total time:', timeElapsed)
    print('Total vehicles passed:', totalVehicles)


def simTime():
    global timeElapsed, simulationTime
    while (True):
        timeElapsed += 1
        time.sleep(1)
        if (timeElapsed == simulationTime):
            showStats()

class Main:
    global allowedVehicleTypesList
    i = 0
    for vehicleType in allowedVehicleTypes:
        if (allowedVehicleTypes[vehicleType]):
            allowedVehicleTypesList.append(i)
        i += 1
    thread1 = threading.Thread(name="initialization", target=initialize, args=())  # initialization
    thread1.daemon = True
    thread1.start()

    # Colours
    black = (0, 0, 0)
    white = (255, 255, 255)

    # Screensize
    screenWidth = 1400
    screenHeight = 800
    screenSize = (screenWidth, screenHeight)

    # Setting background image i.e. image of intersection
    background = pygame.image.load('images/intersection.png')

    screen = pygame.display.set_mode(screenSize)
    pygame.display.set_caption("SIMULATION")

    # Loading signal images and font
    redSignal = pygame.image.load('images/signals/red.png')
    yellowSignal = pygame.image.load('images/signals/yellow.png')
    greenSignal = pygame.image.load('images/signals/green.png')
    font = pygame.font.Font(None, 30)
    thread2 = threading.Thread(name="generateVehicles", target=generateVehicles, args=())  # Generating vehicles
    thread2.daemon = True
    thread2.start()

    thread3 = threading.Thread(name="simTime", target=simTime, args=())
    thread3.daemon = True
    thread3.start()

    while True:
        for event in pygame.event.get():
            if event.type == pygame.QUIT:
                showStats()
                sys.exit()

        screen.blit(background, (0, 0))  # display background in simulation
        for i in range(0,
                       noOfSignals):  # display signal and set timer according to current status: green, yello, or red
            if (i == currentGreen):
                if (currentYellow == 1):
                    signals[i].signalText = signals[i].yellow
                    screen.blit(yellowSignal, signalCoods[i])
                else:
                    signals[i].signalText = signals[i].green
                    screen.blit(greenSignal, signalCoods[i])
            else:
                if (signals[i].red <= 10):
                    signals[i].signalText = signals[i].red
                else:
                    signals[i].signalText = "---"
                screen.blit(redSignal, signalCoods[i])
        signalTexts = ["", "", "", ""]

        # display signal timer
        for i in range(0, noOfSignals):
            signalTexts[i] = font.render(str(signals[i].signalText), True, white, black)
            screen.blit(signalTexts[i], signalTimerCoods[i])


        # display vehicle count
        for i in range(0, noOfSignals):
            displayText = vehicles[directionNumbers[i]]['crossed']
            vehicleCountTexts[i] = font.render(str(displayText), True, black, white)
            screen.blit(vehicleCountTexts[i], vehicleCountCoods[i])

        # display time elapsed
        timeElapsedText = font.render(("Time Elapsed: " + str(timeElapsed)), True, black, white)
        screen.blit(timeElapsedText, timeElapsedCoods)

        # display the vehicles
        for vehicle in simulation:
            screen.blit(vehicle.image, [vehicle.x, vehicle.y])
            vehicle.move()
        pygame.display.update()
Main()

OUTPUT